Practical 8

Aim:- Simulating MPLS environment

Topology:-

Step 1 – IP addressing of MPLS Core and OSPF

R1 Console

conf t

hostname R1

int lo0

ip add 1.1.1.1 255.255.255.255

ip ospf 1 area 0

int f0/0

ip add 10.0.0.1 255.255.255.0

no shut

ip ospf 1 area 0

R2 Console

conf t

hostname R2

int lo0

ip add 2.2.2.2 255.255.255.255

ip ospf 1 are 0

int f0/0

ip add 10.0.0.2 255.255.255.0

no shut

ip ospf 1 area 0

int f0/1

ip add 10.0.1.2 255.255.255.0

no shut

ip ospf 1 area 0

R3 Console

conf t

hostname R3

int lo0

ip add 3.3.3.3 255.255.255.255

ip ospf 1 are 0

int f0/0

ip add 10.0.1.3 255.255.255.0

no shut

ip ospf 1 area 0

R1 Console

Use ping 3.3.3.3 source lo0 command to check full ip connectivity between R1, R2, R3 to verify if we can ping between the loopbacks of R1 and R3

ping 3.3.3.3 source lo0

Step 2 – Configure LDP on all the interfaces in the MPLS Core

Under the ospf process use the mpls ldp autoconfig command this will enable mpls label distribution protocol on every interface running ospf under that specific process.

R1 Console

router ospf 1

mpls ldp autoconfig

R2 Console

router ospf 1

mpls ldp autoconfig

R3 Console

router ospf 1

mpls ldp autoconfig

R2 Console

verify the mpls interfaces with  sh mpls interface command.

verify the LDP neighbors with the sh mpls ldp neighbor command.

One more verification to confirm LDP is running ok is to do a trace between R1 and R3 and verify if we get MPLS Labels show up in the trace.

R1 Console

trace 3.3.3.3

Step 3 – MPLS BGP Configuration between R1 and R3

Establish a Multi Protocol BGP session between R1 and R3 this is done by configuring the vpnv4  address family as below

R1 Console

conf t

router bgp 1

neighbor 3.3.3.3 remote-as 1

neighbor 3.3.3.3 update-source Loopback0

no auto-summary

address-family vpnv4

neighbor 3.3.3.3 activate

R3 Console

conf t

router bgp 1

neighbor 1.1.1.1 remote-as 1

neighbor 1.1.1.1 update-source Loopback0

no auto-summary

address-family vpnv4

neighbor 1.1.1.1 activate

To verify the BGP session between R1 and R3 issue the command sh bgp vpnv4 unicast all summary

R1 Console

sh bgp vpnv4 unicast all summary

Step 4 – Add two more routers, create VRFs

Add two more routers into the topology.

R4 Console

int lo0

ip add 4.4.4.4 255.255.255.255

ip ospf 2 area 2

int f0/0

ip add 192.168.1.4 255.255.255.0

ip ospf 2 area 2

no shut

R1 Console

int f0/1

no shut

ip add 192.168.1.1 255.255.255.0

Now create VRF on R1

R1 Console

ip vrf RED

rd 4:4

route-target both 4:4

So now we have configured the VRF on R1 we need to move the interface F0/1 into that VRF

int f0/1

ip vrf forwarding RED

Just reapply below

R1 Console

int f0/1

ip address 192.168.1.1 255.255.255.0

Now view the config on R1 int f0/1 we can see the VRF configured

R1 Console

sh run int f0/1

Now that there are 2 routing tables within R1

- The Global Routing Table

- The Routing Table for VRF RED

Issue the command sh ip route this shows the routes in the global table and you will notice that you do not see 192.168.1.0/24

R1 Console

sh ip route

Issue the command sh ip route vrf red – this will show the routes in the routing table for VRF RED

Enable OSPF on this interface and get the loopback address for R4 in the VRF RED routing table.

R1 Console

int f0/1

ip ospf 2 area 2

Check the routes in the VRF RED routing table we should see 4.4.4.4 in there as well.

sh ip route vrf RED

Repeat this process for R3 and R6 as below:

R6 Console

int lo0

ip add 6.6.6.6 255.255.255.255

ip ospf 2 area 2

int f0/0

ip add 192.168.2.6 255.255.255.0

ip ospf 2 area 2

no shut

R3 Console

int f0/1

no shut

ip add 192.168.2.3 255.255.255.0

create VRF on R3

R3 Console

ip vrf RED

rd 4:4

route-target both 4:4

int f0/1

ip vrf forwarding RED

int f0/1

ip address 192.168.2.1 255.255.255.0

sh run int f0/1

int f0/1

ip ospf 2 area 2

sh ip route vrf RED

Perform verification on R1 and R4

R1 and R4 Console

sh ip route

R1 Console

sh ip route vrf RED

Step - Redistribute OSPF into MP-BGP on R1

R1 Console

router bgp 1

address-family ipv4 vrf RED

redistribute ospf 2

Step- Redistribute OSPF into MP-BGP on R3

R3 Console

router bgp 1

address-family ipv4 vrf RED

redistribute ospf 2

This has enabled redistribution of the OSPF routes into BGP. We can check the routes from R4 and R6 are now showing in the BGP table for their VRF with below command

R1 and R3 Console

sh ip bgp vpnv4 vrf RED

The final step is to get the routes that have come across the MPLS back into OSPF and then we can get end to end connectivity.

R1 and R3 Console

router ospf 2

redistribute bgp 1 subnets

If all has worked we should be now able to ping 6.6.6.6 from R4

R4 and R6 Console

sh ip route

we have 4.4.4.4 in there so we should be able to ping across the MPLS

R4 Console

ping 6.6.6.6

Add below command in R4 and R6 Console:

router ospf 2

mpls ldp autoconfig

R4 Console

After that perform trace we can able to see lables

trace 6.6.6.6